SNMP exercises, part 1

PacNOG5, Papeete, PF

1. Getting packages (should already be installed)

> apt-get install snmp

> apt-get install snmpd

> apt-get install mbrowse

2. GET and WALK

To control that your SNMP installation works:

- The backbone router and net routers

> snmpstatus -c pacn0g2k9 -v2c 192.168.1.221

> snmpstatus -c pacn0g2k9 -v2c 192.168.1.222

- The NOC server

> snmpstatus -c pacn0g2k9 -v2c 192.168.1.224

- The network switches:

> snmpstatus -c pacn0g2k9 -v2c 192.168.1.223

> snmpstatus -c pacn0g2k9 -v2c 192.168.2.125

> snmpstatus -c pacn0g2k9 -v2c 192.168.2.253

- Try to snmpwalk different parts of these equipments' MIBs:

> snmpwalk -c pacn0g2k9 -v2c 192.168.1.Y 1.3.6.1.4.1.9.9.13.1.3 | more

> ...

a) Do all the devices answer ?

b) Do you notice anything important about the OID on the output ?

3. Configuration of snmpd

- Edit the following file:

> vi /etc/snmp/snmpd.conf

Comment the line (ADD '#' in front):

com2sec paranoid  default         public

... so that it becomes:

#com2sec paranoid  default         public

And UNcomment the line (REMOVE the '#' in front) and change community:

#com2sec readonly  default         public

... so that it becomes:

com2sec readonly  default         pacn0g2k9

Edit the file /etc/default/snmpd, and find the line:

SNMPDOPTS='-Lsd -Lf /dev/null -u snmp -I -smux -p /var/run/snmpd.pid 127.0.0.1'

Remove 127.0.0.1 at the end, so you have:

SNMPDOPTS='-Lsd -Lf /dev/null -u snmp -I -smux -p /var/run/snmpd.pid'

- Restart snmpd

> /etc/init.d/snmpd stop

> /etc/init.d/snmpd start

4. Check that snmpd is working:

> snmpstatus -c pacn0g2k9 -v2c localhost

- What do you observe ?

5. Check now that you can run snmpstatus against your neighbor's server:

- Find out what your neighbor's IP is, ask them to run:

> ifconfig

(your IP is 192.168.2.X where X is the IP of the PC of your neighbor,

for example: 2.101, 2.102, 2.201, 2.203, ...)

- Check snmp against their machine:

> snmpstatus -c public -v2c 192.168.2.10x

or

> snmpstatus -c public -v2c 192.168.2.20x

6. SNMPwalk – the rest of MIB-II

- Try and run snmpwalk on the routers, switches, and other hosts in the

network:

> snmpwalk -c pacn0g2k9 -v2c 192.168.1.221 (and .222, .223, .224)

> snmpwalk -c pacn0g2k9 -v2c 192.168.2.X

Note the kind of information you can obtain.

> snmpwalk -c pacn0g2k9 -v2c 192.168.2.X ifDescr

> snmpwalk -c pacn0g2k9 -v2c 192.168.2.X ifTable

> snmpwalk -c pacn0g2k9 -v2c 192.168.2.X ifDescr

> snmpwalk -c pacn0g2k9 -v2c 192.168.2.X ifOperStatus

> snmpwalk -c pacn0g2k9 -v2c 192.168.2.X ifAdminStatus

> snmpwalk -c pacn0g2k9 -v2c 192.168.2.X if

7. Adding MIBs

Remember when you ran:

> snmpwalk -c pacn0g2k9 -v2c 192.168.1.221  1.3.6.1.4.1.9.9.13.1.3 | more

If you noticed, the SNMP client (snmpwalk) couldn't interpret

all the OIDs coming back from the Agent:

SNMPv2-SMI::enterprises.9.9.13.1.3.1.2.1 = STRING: "chassis"

SNMPv2-SMI::enterprises.9.9.13.1.3.1.6.1 = INTEGER: 1

or

SNMPv2-SMI::enterprises.9.9.13.1.3.1.2.1 = STRING: "Temp at inlet"

SNMPv2-SMI::enterprises.9.9.13.1.3.1.2.2 = STRING: "Temp at outlet"

SNMPv2-SMI::enterprises.9.9.13.1.3.1.3.1 = Gauge32: 21

SNMPv2-SMI::enterprises.9.9.13.1.3.1.3.2 = Gauge32: 30

SNMPv2-SMI::enterprises.9.9.13.1.3.1.4.1 = INTEGER: 60

SNMPv2-SMI::enterprises.9.9.13.1.3.1.4.2 = INTEGER: 72

SNMPv2-SMI::enterprises.9.9.13.1.3.1.6.1 = INTEGER: 1

SNMPv2-SMI::enterprises.9.9.13.1.3.1.6.2 = INTEGER: 1

What is '9.9.13.1.3.1.3' ?

To be able to interpret this information, we need to download extra

mibs...

- Download the following files to your machine:

ftp://ftp.cisco.com/pub/mibs/v2/CISCO-SMI.my

ftp://ftp.cisco.com/pub/mibs/v2/CISCO-ENVMON-MIB.my

> cd /usr/share/snmp/mibs

> wget ftp://ftp.cisco.com/pub/mibs/v2/CISCO-SMI.my

> wget ftp://ftp.cisco.com/pub/mibs/v2/CISCO-ENVMON-MIB.my

- Create the file /usr/share/snmp/snmp.conf, and put into it:

mibdirs /usr/share/snmp/mibs

mibs ALL

This tells the snmp* commands that they should load ALL mibs in the
    mibdir /usr/share/snmp/mibs

Save the file, quit.

Now, try again:

> snmpwalk -c pacn0g2k9 -v2c 192.168.1.221  1.3.6.1.4.1.9.9.13.1.3 | more

What do you notice ?